# SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1 - EXPERIMENTS (ANNEXURE A) — WORKED SOLUTIONS

| Course Code | CSA1205 |
| --- | --- |
| Course Title | Computer Architecture for Multicore Systems |
| CO Assessed | CO3 |
| Assessment | ASSESSMENT TOOL 1 - EXPERIMENTS |
| Weightage | 40% |
| Total Marks | 25 |

CO3 Statement: Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6)

NAME: A GUNA SEKAR

REG NO:192525302

# Experiment 1: Register Transfer Operations and ALU (ADD, SUB, AND, OR)

## Aim

To design and simulate a digital system that performs register transfer operations and basic arithmetic/logic operations, and to analyze the role of the ALU and the bus in moving and processing data.

## Theory / Background

- A digital computer consists of a set of registers (R0-R3) connected through a common bus.

- Register Transfer Language (RTL) notation, e.g. R2 <- R0, describes the movement of data between registers each clock cycle.

- The Arithmetic Logic Unit (ALU) is the functional unit that performs ADD, SUB, AND, OR and other operations; its two inputs come from registers via the bus and its output is written back to a destination register via the same bus.

- Every transfer or ALU operation is assumed to take one clock cycle on the shared bus.

## Design / Algorithm

1. Model each register as an object holding a 16-bit value.

2. Model the ALU as a stateless function execute(op, a, b) implementing ADD, SUB, AND, OR.

3. Implement transfer(src, dst) for plain register-to-register transfer through the bus.

4. Implement alu_op(op, src1, src2, dst) to read two source registers, invoke the ALU, and write the result to the destination register.

5. Log every operation with the cycle number, the values on the bus, and print the register file before and after the sequence.

## Simulation Code (Python)

"""

Experiment 1: Register Transfer Language (RTL) Simulator with ALU

Demonstrates: register-to-register transfer, ADD, SUB, AND, OR operations,

and the role of the ALU as the functional unit that processes data

moving between registers over a common bus.

"""

class Register:

def __init__(self, name, value=0):

self.name = name

self.value = value & 0xFFFF   # 16-bit register

def __repr__(self):

return f"{self.name}={self.value:016b} ({self.value})"

class ALU:

"""Arithmetic Logic Unit: performs operations on two operands."""

@staticmethod

def execute(op, a, b):

if op == "ADD":

result = (a + b) & 0xFFFF

elif op == "SUB":

result = (a - b) & 0xFFFF

elif op == "AND":

result = a & b

elif op == "OR":

result = a | b

else:

raise ValueError(f"Unknown ALU operation: {op}")

return result

class RegisterFile:

"""Set of general purpose registers connected by a common bus."""

def __init__(self, names):

self.regs = {n: Register(n) for n in names}

self.bus_log = []

self.cycle = 0

def load(self, name, value):

self.regs[name].value = value & 0xFFFF

def transfer(self, src, dst):

"""Register Transfer: dst <- src (goes through the bus)."""

self.cycle += 1

value = self.regs[src].value

self.regs[dst].value = value

self.bus_log.append(

f"Cycle {self.cycle:2d} | RT : {dst} <- {src}  (Bus carries {value})"

)

def alu_op(self, op, src1, src2, dst):

"""dst <- src1 OP src2, performed by the ALU, result written back via bus."""

self.cycle += 1

a, b = self.regs[src1].value, self.regs[src2].value

result = ALU.execute(op, a, b)

self.regs[dst].value = result

self.bus_log.append(

f"Cycle {self.cycle:2d} | ALU: {dst} <- {src1} {op} {src2}  "

f"({a} {op} {b} = {result})"

)

def dump(self):

return " | ".join(str(r) for r in self.regs.values())

if __name__ == "__main__":

rf = RegisterFile(["R0", "R1", "R2", "R3"])

# Initial load of operands into registers

rf.load("R0", 25)

rf.load("R1", 10)

print("Initial register state:")

print(rf.dump())

print()

print("Executing sequence of Register Transfer + ALU operations:")

rf.transfer("R0", "R2")          # simple register-to-register transfer

rf.alu_op("ADD", "R0", "R1", "R3")   # R3 <- R0 + R1

rf.alu_op("SUB", "R0", "R1", "R2")   # R2 <- R0 - R1

rf.alu_op("AND", "R0", "R1", "R1")   # R1 <- R0 AND R1

rf.alu_op("OR",  "R2", "R3", "R0")   # R0 <- R2 OR R3

for line in rf.bus_log:

print(line)

print()

print("Final register state:")

print(rf.dump())

print(f"\nTotal bus cycles used: {rf.cycle}")

## Sample Output

Initial register state:

R0=0000000000011001 (25) | R1=0000000000001010 (10) | R2=0000000000000000 (0) | R3=0000000000000000 (0)

Executing sequence of Register Transfer + ALU operations:

Cycle  1 | RT : R2 <- R0  (Bus carries 25)

Cycle  2 | ALU: R3 <- R0 ADD R1  (25 ADD 10 = 35)

Cycle  3 | ALU: R2 <- R0 SUB R1  (25 SUB 10 = 15)

Cycle  4 | ALU: R1 <- R0 AND R1  (25 AND 10 = 8)

Cycle  5 | ALU: R0 <- R2 OR R3  (15 OR 35 = 47)

Final register state:

R0=0000000000101111 (47) | R1=0000000000001000 (8) | R2=0000000000001111 (15) | R3=0000000000100011 (35)

Total bus cycles used: 5

## Result & Analysis

- The simulation confirms that a plain register transfer (R2 <- R0) takes exactly one bus cycle and does not involve the ALU.

- Each ALU operation (ADD, SUB, AND, OR) also takes one cycle: two operands are placed on the bus into the ALU, and the single-cycle result is written back to the destination register.

- 5 operations (1 transfer + 4 ALU operations) consumed exactly 5 bus cycles, confirming that in a single-bus organization only one register transfer/ALU result can use the bus per cycle.

- The ALU is the central processing element: every arithmetic/logic instruction must route its operands and result through it, making it a potential performance bottleneck if it is used every cycle.

# Experiment 2: Single-Bus vs Multi-Bus Computer Organization

## Aim

To model a computer system using single-bus and multi-bus organization, perform data transfers between registers, memory and I/O, and compare the time taken and efficiency of both approaches.

## Theory / Background

- In a single-bus organization, all units (registers, ALU, memory, I/O) share one common data path, so only one data transfer can occur per clock cycle.

- In a multi-bus organization (e.g., a 3-bus system with A-bus, B-bus, C-bus), multiple independent transfers can be issued in the same cycle as long as they use different buses and do not conflict on the same destination.

- Multi-bus systems reduce bottlenecks because the ALU can, for example, read two operands from two buses and write its result to a third bus in the same cycle, instead of needing three separate cycles.

## Design / Algorithm

1. Define a common workload of 6 independent register/memory/I/O transfers.

2. SingleBusSystem.transfer(): executes exactly one transfer per call/cycle.

3. MultiBusSystem.transfer_batch(): groups transfers into batches of up to 3 (NUM_BUSES) and executes each batch in a single cycle.

4. Run the same workload on both models, log the cycle each transfer completes in, and compute total cycles, throughput (transfers/cycle), and speedup.

## Simulation Code (Python)

"""

Experiment 2: Single-Bus vs Multi-Bus Computer Organization

Models data transfer between Registers, Memory and I/O using

(a) a single shared bus  -> only ONE transfer can happen per cycle

(b) a three-bus system   -> up to THREE independent transfers per cycle

and compares the time (cycles) taken and effective throughput.

"""

class Memory(dict):

def read(self, addr):

return self.get(addr, 0)

def write(self, addr, value):

self[addr] = value

class SingleBusSystem:

"""All units (registers, memory, I/O, ALU) share ONE common bus.

Only one data transfer can use the bus in a given cycle, so

dependent transfers must be serialized."""

def __init__(self):

self.regs = {"R0": 0, "R1": 0, "R2": 0}

self.mem = Memory()

self.io = {}

self.cycles = 0

self.log = []

def transfer(self, src_kind, src, dst_kind, dst):

self.cycles += 1

value = self._read(src_kind, src)

self._write(dst_kind, dst, value)

self.log.append(f"[Single-Bus] Cycle {self.cycles:2d}: {dst_kind}:{dst} <- {src_kind}:{src} "

f"(value={value})  [bus busy]")

def _read(self, kind, name):

return self.regs[name] if kind == "REG" else (self.mem.read(name) if kind == "MEM" else self.io.get(name, 0))

def _write(self, kind, name, value):

if kind == "REG":

self.regs[name] = value

elif kind == "MEM":

self.mem.write(name, value)

else:

self.io[name] = value

class MultiBusSystem:

"""Three independent buses (e.g., A-bus, B-bus, C-bus) allow up to

3 register/memory/IO transfers to be issued in the SAME cycle,

provided they don't target the same destination."""

NUM_BUSES = 3

def __init__(self):

self.regs = {"R0": 0, "R1": 0, "R2": 0}

self.mem = Memory()

self.io = {}

self.cycles = 0

self.log = []

def _read(self, kind, name):

return self.regs[name] if kind == "REG" else (self.mem.read(name) if kind == "MEM" else self.io.get(name, 0))

def _write(self, kind, name, value):

if kind == "REG":

self.regs[name] = value

elif kind == "MEM":

self.mem.write(name, value)

else:

self.io[name] = value

def transfer_batch(self, transfers):

"""transfers: list of (src_kind, src, dst_kind, dst); up to NUM_BUSES

of them execute together in a single cycle."""

self.cycles += 1

for i in range(0, len(transfers), self.NUM_BUSES):

group = transfers[i:i + self.NUM_BUSES]

if i > 0:

self.cycles += 1

for (sk, s, dk, d) in group:

value = self._read(sk, s)

self._write(dk, d, value)

self.log.append(f"[Multi-Bus ] Cycle {self.cycles:2d}: {dk}:{d} <- {sk}:{s} "

f"(value={value})  [bus {group.index((sk,s,dk,d))+1}]")

if __name__ == "__main__":

# Common workload: 6 independent data movements between

# registers, memory, and I/O

workload = [

("REG", "R0", "MEM", 100),

("MEM", 100, "REG", "R1"),

("IO", "PORT1", "REG", "R2"),

("REG", "R1", "MEM", 101),

("REG", "R2", "IO", "PORT2"),

("MEM", 101, "REG", "R0"),

]

# ---- Single bus system: strictly one transfer per cycle ----

sb = SingleBusSystem()

sb.regs["R0"] = 55

sb.io["PORT1"] = 77

for (sk, s, dk, d) in workload:

sb.transfer(sk, s, dk, d)

# ---- Multi bus system: up to 3 transfers per cycle ----

mb = MultiBusSystem()

mb.regs["R0"] = 55

mb.io["PORT1"] = 77

mb.transfer_batch(workload)

print("\n".join(sb.log))

print()

print("\n".join(mb.log))

print("\n--- Comparison ---")

print(f"Single-bus system : {sb.cycles} cycles for {len(workload)} transfers "

f"-> throughput = {len(workload)/sb.cycles:.2f} transfers/cycle")

print(f"Multi-bus system  : {mb.cycles} cycles for {len(workload)} transfers "

f"-> throughput = {len(workload)/mb.cycles:.2f} transfers/cycle")

speedup = sb.cycles / mb.cycles

print(f"Speedup of multi-bus over single-bus: {speedup:.2f}x")

## Sample Output

[Single-Bus] Cycle  1: MEM:100 <- REG:R0 (value=55)  [bus busy]

[Single-Bus] Cycle  2: REG:R1 <- MEM:100 (value=55)  [bus busy]

[Single-Bus] Cycle  3: REG:R2 <- IO:PORT1 (value=77)  [bus busy]

[Single-Bus] Cycle  4: MEM:101 <- REG:R1 (value=55)  [bus busy]

[Single-Bus] Cycle  5: IO:PORT2 <- REG:R2 (value=77)  [bus busy]

[Single-Bus] Cycle  6: REG:R0 <- MEM:101 (value=55)  [bus busy]

[Multi-Bus ] Cycle  1: MEM:100 <- REG:R0 (value=55)  [bus 1]

[Multi-Bus ] Cycle  1: REG:R1 <- MEM:100 (value=55)  [bus 2]

[Multi-Bus ] Cycle  1: REG:R2 <- IO:PORT1 (value=77)  [bus 3]

[Multi-Bus ] Cycle  2: MEM:101 <- REG:R1 (value=55)  [bus 1]

[Multi-Bus ] Cycle  2: IO:PORT2 <- REG:R2 (value=77)  [bus 2]

[Multi-Bus ] Cycle  2: REG:R0 <- MEM:101 (value=55)  [bus 3]

--- Comparison ---

Single-bus system : 6 cycles for 6 transfers -> throughput = 1.00 transfers/cycle

Multi-bus system  : 2 cycles for 6 transfers -> throughput = 3.00 transfers/cycle

Speedup of multi-bus over single-bus: 3.00x

## Result & Analysis

- The single-bus system needed 6 cycles for 6 transfers (throughput = 1.00 transfers/cycle) because every transfer had to wait its turn for the shared bus.

- The multi-bus system completed the same 6 transfers in only 2 cycles (throughput = 3.00 transfers/cycle) by issuing up to 3 independent transfers per cycle.

- This gives a 3x speedup for the multi-bus organization on this workload, directly demonstrating how multiple parallel buses reduce bus-contention bottlenecks.

- The trade-off is hardware cost/complexity: a multi-bus design needs more physical wiring, multiplexers/drivers, and control logic than a single shared bus.

# Experiment 3: 5-Stage Instruction Pipeline (IF, ID, EX, MEM, WB)

## Aim

To simulate a 5-stage instruction pipeline, execute a set of instructions, and measure speedup and throughput relative to a non-pipelined (sequential) execution model.

## Theory / Background

- Classic RISC pipeline stages: Instruction Fetch (IF), Instruction Decode (ID), Execute (EX), Memory access (MEM), Write-back (WB).

- In an ideal pipeline, a new instruction is fetched every cycle, so N instructions overlap in different stages.

- For N instructions and k stages, ideal pipelined execution takes k + (N-1) cycles, versus N*k cycles for a strictly sequential (non-pipelined) machine where each instruction must finish all stages before the next starts.

- Speedup = (non-pipelined cycles) / (pipelined cycles); it approaches the number of stages (k) as N becomes large.

## Design / Algorithm

1. Represent the pipeline as a list of 5 stage names.

2. simulate_pipeline(N): compute, for every instruction, which cycle occupies which stage, and build an occupancy chart.

3. simulate_non_pipelined(N): total cycles = N * (number of stages).

4. Print the occupancy chart and compute speedup and throughput (instructions/cycle) for both models.

## Simulation Code (Python)

"""

Experiment 3: 5-Stage Instruction Pipeline Simulator (IF, ID, EX, MEM, WB)

Measures speedup and throughput of a pipelined datapath compared

to a non-pipelined (sequential) datapath for the same instruction stream.

"""

STAGES = ["IF", "ID", "EX", "MEM", "WB"]

def simulate_pipeline(num_instructions):

"""Ideal 5-stage pipeline, one instruction issued per cycle.

Total cycles = num_stages + (num_instructions - 1)"""

n = len(STAGES)

total_cycles = n + (num_instructions - 1)

# Build cycle x instruction occupancy chart

chart = []

for instr in range(1, num_instructions + 1):

row = [""] * total_cycles

for s_idx, stage in enumerate(STAGES):

cycle = instr + s_idx  # 1-indexed cycle this stage occupies

row[cycle - 1] = stage

chart.append(row)

return total_cycles, chart

def simulate_non_pipelined(num_instructions):

"""Each instruction must fully complete all 5 stages before the

next instruction begins -> total cycles = instructions * stages"""

return num_instructions * len(STAGES)

def print_chart(chart, total_cycles):

header = "Instr\\Cyc | " + " | ".join(f"{c+1:3d}" for c in range(total_cycles))

print(header)

print("-" * len(header))

for i, row in enumerate(chart, start=1):

cells = " | ".join(f"{s:3s}" if s else "   " for s in row)

print(f"  I{i:<6}| {cells}")

if __name__ == "__main__":

num_instructions = 6

pipe_cycles, chart = simulate_pipeline(num_instructions)

seq_cycles = simulate_non_pipelined(num_instructions)

print(f"Pipeline occupancy chart for {num_instructions} instructions "

f"through a 5-stage pipeline (IF, ID, EX, MEM, WB):\n")

print_chart(chart, pipe_cycles)

speedup = seq_cycles / pipe_cycles

ideal_speedup = len(STAGES)

throughput_pipe = num_instructions / pipe_cycles

throughput_seq = num_instructions / seq_cycles

print("\n--- Performance Analysis ---")

print(f"Non-pipelined total cycles : {seq_cycles}  "

f"(throughput = {throughput_seq:.3f} instr/cycle)")

print(f"Pipelined total cycles     : {pipe_cycles}  "

f"(throughput = {throughput_pipe:.3f} instr/cycle)")

print(f"Speedup (non-pipelined/pipelined) : {speedup:.2f}x")

print(f"Theoretical max speedup (=#stages): {ideal_speedup}x "

f"(approached as num_instructions -> infinity)")

## Sample Output

Pipeline occupancy chart for 6 instructions through a 5-stage pipeline (IF, ID, EX, MEM, WB):

Instr\Cyc |   1 |   2 |   3 |   4 |   5 |   6 |   7 |   8 |   9 |  10

---------------------------------------------------------------------

I1     | IF  | ID  | EX  | MEM | WB  |     |     |     |     |

I2     |     | IF  | ID  | EX  | MEM | WB  |     |     |     |

I3     |     |     | IF  | ID  | EX  | MEM | WB  |     |     |

I4     |     |     |     | IF  | ID  | EX  | MEM | WB  |     |

I5     |     |     |     |     | IF  | ID  | EX  | MEM | WB  |

I6     |     |     |     |     |     | IF  | ID  | EX  | MEM | WB

--- Performance Analysis ---

Non-pipelined total cycles : 30  (throughput = 0.200 instr/cycle)

Pipelined total cycles     : 10  (throughput = 0.600 instr/cycle)

Speedup (non-pipelined/pipelined) : 3.00x

Theoretical max speedup (=#stages): 5x (approached as num_instructions -> infinity)

## Result & Analysis

- For 6 instructions, the non-pipelined model required 30 cycles (throughput 0.20 IPC) while the pipelined model required only 10 cycles (throughput 0.60 IPC).

- This gives a measured speedup of 3.00x for only 6 instructions; the theoretical maximum speedup equals the number of pipeline stages (5x) and is approached as more instructions are executed, since the 4-cycle fill/drain overhead is amortized.

- The occupancy chart shows that from cycle 5 onward, all 5 stages are simultaneously busy with different instructions - this is the core benefit of pipelining: increased instruction throughput without increasing per-instruction latency.

- Pipelining does not reduce the latency of a single instruction (it still takes 5 cycles to complete); it increases overall throughput by overlapping independent instructions.

# Experiment 4: Pipeline Hazards: Data, Control and Structural

## Aim

To develop a pipeline execution scenario in which data, control and structural hazards occur, observe their effect on execution, and implement stalling, data forwarding and branch prediction to resolve them.

## Theory / Background

- Data hazard (RAW - Read After Write): an instruction needs a register value that a nearby preceding instruction has not yet written back.

- Load-use hazard: a special case of data hazard where the needed value is only available after the MEM stage (i.e., a LOAD), so forwarding alone cannot fully avoid a 1-cycle stall.

- Control hazard: caused by branch instructions - the pipeline does not know which instruction to fetch next until the branch is resolved.

- Structural hazard: occurs when two instructions in the pipeline need the same hardware resource (e.g., a single memory port used by both IF and MEM) in the same cycle.

## Design / Algorithm

1. Represent a small program as a list of Instr objects with opcode, destination register, source registers, and flags for branch/memory use.

2. detect_data_hazards(): scan each instruction against its 2 immediate predecessors for a shared register (RAW).

3. simulate_with_forwarding(): resolve RAW hazards using EX/MEM-to-EX forwarding; flag load-use hazards as needing 1 stall cycle even with forwarding.

4. simulate_control_hazard(): apply a predict-not-taken branch predictor; flag a 2-cycle flush penalty whenever the prediction is wrong.

5. simulate_structural_hazard(): detect when more than one instruction needs the shared MEM/IF memory port and propose stalling or a Harvard (separate I-mem/D-mem) architecture as the fix.

## Simulation Code (Python)

"""

Experiment 4: Pipeline Hazard Simulation - Data, Control, Structural

Demonstrates detection of hazards in a 5-stage pipeline (IF, ID, EX, MEM, WB)

and resolves them using:

- Data hazard      -> Data Forwarding (and stalling as fallback)

- Control hazard    -> Branch Prediction (predict-not-taken) + flush on misprediction

- Structural hazard  -> Stalling due to a shared functional unit / memory port

"""

class Instr:

def __init__(self, text, opcode, dst=None, src=None, is_branch=False, uses_mem=False):

self.text = text

self.opcode = opcode

self.dst = dst

self.src = src or []

self.is_branch = is_branch

self.uses_mem = uses_mem

def detect_data_hazards(program):

print("== DATA HAZARD DETECTION (RAW) ==")

for i in range(1, len(program)):

cur = program[i]

for j in range(max(0, i - 2), i):

prev = program[j]

if prev.dst and prev.dst in cur.src:

gap = i - j

print(f"  RAW hazard: '{cur.text}' needs {prev.dst} produced by '{prev.text}' "

f"({gap} instruction(s) apart)")

print()

def simulate_with_forwarding(program):

"""With forwarding, EX result of instruction i-1 is forwarded to EX of

instruction i, so no stall is needed unless it's a load-use hazard."""

print("== RESOLUTION: DATA FORWARDING ==")

stalls = 0

for i in range(1, len(program)):

cur, prev = program[i], program[i - 1]

if prev.dst and prev.dst in cur.src:

if prev.uses_mem:  # load-use hazard: data not ready until MEM stage

stalls += 1

print(f"  Load-use hazard between '{prev.text}' and '{cur.text}' "

f"-> 1 stall cycle inserted even with forwarding")

else:

print(f"  '{prev.text}' -> forwarded directly from EX/MEM to EX of "

f"'{cur.text}' (0 stalls)")

print(f"  Total stall cycles introduced: {stalls}\n")

return stalls

def simulate_control_hazard(program):

print("== CONTROL HAZARD: BRANCH PREDICTION ==")

for i, ins in enumerate(program):

if ins.is_branch:

predicted_taken = False  # simple predict-not-taken strategy

actual_taken = ins.text.endswith("(taken)")

print(f"  Branch '{ins.text}' at instruction {i+1}: predicted "

f"{'TAKEN' if predicted_taken else 'NOT TAKEN'}, "

f"actual {'TAKEN' if actual_taken else 'NOT TAKEN'}")

if predicted_taken != actual_taken:

print("    -> Misprediction! Pipeline flush: 2 cycles wasted "

"(instructions fetched after branch discarded)")

else:

print("    -> Correct prediction: 0 cycles wasted")

print()

def simulate_structural_hazard(program):

print("== STRUCTURAL HAZARD: SHARED MEMORY PORT ==")

mem_stage_users = [ins for ins in program if ins.uses_mem]

print(f"  {len(mem_stage_users)} instruction(s) require the MEM stage "

f"(shared with IF for instruction fetch on a single memory port).")

if len(mem_stage_users) > 1:

print("  -> Structural conflict possible when a LOAD/STORE's MEM stage "

"overlaps with another instruction's IF stage.")

print("  -> Resolution: stall the IF of the conflicting instruction for 1 cycle "

"(or use separate instruction/data memories - Harvard architecture).")

print()

if __name__ == "__main__":

program = [

Instr("ADD R1, R2, R3", "ADD", dst="R1", src=["R2", "R3"]),

Instr("SUB R4, R1, R5", "SUB", dst="R4", src=["R1", "R5"]),   # RAW on R1

Instr("LOAD R6, 0(R7)", "LOAD", dst="R6", src=["R7"], uses_mem=True),

Instr("AND R8, R6, R9", "AND", dst="R8", src=["R6", "R9"]),   # load-use hazard on R6

Instr("BEQ R8, R0, LABEL (taken)", "BEQ", src=["R8", "R0"], is_branch=True),

Instr("OR  R10, R4, R1", "OR", dst="R10", src=["R4", "R1"]),

Instr("STORE R10, 4(R7)", "STORE", src=["R10", "R7"], uses_mem=True),

]

detect_data_hazards(program)

simulate_with_forwarding(program)

simulate_control_hazard(program)

simulate_structural_hazard(program)

print("== SUMMARY ==")

print("Data hazards      -> resolved mostly by forwarding; load-use hazards still need 1 stall")

print("Control hazards   -> resolved by branch prediction; mispredictions cost a pipeline flush")

print("Structural hazards -> resolved by stalling or duplicating the contended resource")

## Sample Output

== DATA HAZARD DETECTION (RAW) ==

RAW hazard: 'SUB R4, R1, R5' needs R1 produced by 'ADD R1, R2, R3' (1 instruction(s) apart)

RAW hazard: 'AND R8, R6, R9' needs R6 produced by 'LOAD R6, 0(R7)' (1 instruction(s) apart)

RAW hazard: 'BEQ R8, R0, LABEL (taken)' needs R8 produced by 'AND R8, R6, R9' (1 instruction(s) apart)

RAW hazard: 'STORE R10, 4(R7)' needs R10 produced by 'OR  R10, R4, R1' (1 instruction(s) apart)

== RESOLUTION: DATA FORWARDING ==

'ADD R1, R2, R3' -> forwarded directly from EX/MEM to EX of 'SUB R4, R1, R5' (0 stalls)

Load-use hazard between 'LOAD R6, 0(R7)' and 'AND R8, R6, R9' -> 1 stall cycle inserted even with forwarding

'AND R8, R6, R9' -> forwarded directly from EX/MEM to EX of 'BEQ R8, R0, LABEL (taken)' (0 stalls)

'OR  R10, R4, R1' -> forwarded directly from EX/MEM to EX of 'STORE R10, 4(R7)' (0 stalls)

Total stall cycles introduced: 1

== CONTROL HAZARD: BRANCH PREDICTION ==

Branch 'BEQ R8, R0, LABEL (taken)' at instruction 5: predicted NOT TAKEN, actual TAKEN

-> Misprediction! Pipeline flush: 2 cycles wasted (instructions fetched after branch discarded)

== STRUCTURAL HAZARD: SHARED MEMORY PORT ==

2 instruction(s) require the MEM stage (shared with IF for instruction fetch on a single memory port).

-> Structural conflict possible when a LOAD/STORE's MEM stage overlaps with another instruction's IF stage.

-> Resolution: stall the IF of the conflicting instruction for 1 cycle (or use separate instruction/data memories - Harvard architecture).

== SUMMARY ==

Data hazards      -> resolved mostly by forwarding; load-use hazards still need 1 stall

Control hazards   -> resolved by branch prediction; mispredictions cost a pipeline flush

Structural hazards -> resolved by stalling or duplicating the contended resource

## Result & Analysis

- 4 RAW data hazards were detected in the 7-instruction program; 3 of them were fully resolved with 0 extra cycles via forwarding, while the LOAD -> AND load-use hazard still required 1 stall cycle.

- The BEQ branch was predicted 'not taken' but was actually taken, causing a misprediction that flushed 2 already-fetched instructions from the pipeline - illustrating the real cost of control hazards.

- A structural hazard was identified between the 2 memory-using instructions (LOAD/STORE) and instruction fetch competing for the single memory port; the fix proposed is stalling IF for 1 cycle or moving to a Harvard architecture with separate instruction and data memories.

- Overall, forwarding eliminates most data-hazard stalls, but load-use hazards, branch mispredictions, and shared-resource conflicts still introduce unavoidable pipeline bubbles that reduce the ideal speedup measured in Experiment 3.

# Experiment 5: Superscalar, Out-of-Order, Speculative Execution and SMT

## Aim

To analyze a processor supporting superscalar execution (including out-of-order and speculative execution) and extend the study to simultaneous multithreading (SMT), evaluating instruction throughput and resource utilization.

## Theory / Background

- Superscalar execution: the processor can fetch, decode and issue more than one instruction per cycle (issue width > 1) by duplicating functional units.

- In-order superscalar: instructions must still issue in their original program order, so a dependency between two adjacent instructions can prevent them from issuing together, wasting an issue slot.

- Out-of-order (OoO) execution: instructions are held in an instruction window/reservation stations and issued as soon as their operands are ready, regardless of program order (enabled by register renaming), maximizing use of the available issue width.

- Speculative execution: branches are predicted, and instructions past the branch are fetched and executed speculatively; if the prediction is correct the work is kept, otherwise it is squashed (as in Experiment 4).

- Simultaneous Multithreading (SMT)/Hyper-Threading: a single superscalar core interleaves instructions from 2 or more independent hardware threads in the same cycle, so a stall (bubble) in one thread's dependency chain can be filled by ready instructions from another thread, improving overall functional-unit utilization.

## Design / Algorithm

1. scalar_inorder(): baseline of 1 instruction retired per cycle.

2. superscalar_inorder(width=2): issue up to 2 instructions per cycle strictly in program order; stop the group early if an in-group dependency is found.

3. superscalar_ooo_speculative(width=2): each cycle, scan the whole remaining instruction window for up to 2 instructions whose source operands are already available (out of program order), issue them, and mark their destinations as available for the next cycle - modelling reservation-station based OoO issue with renamed registers.

4. smt_shared_core(): merge 2 independent threads onto one 2-wide core, round-robin filling issue slots each cycle from whichever thread has a ready instruction, so one thread's dependency stall is masked by the other thread's independent work.

5. Compare cycles and instructions-per-cycle (IPC) across all four models on the same/comparable instruction streams.

## Simulation Code (Python)

"""

Experiment 5: Superscalar Execution, Out-of-Order (OoO) Execution,

Speculative Execution, and Simultaneous Multithreading (SMT)

Compares instruction throughput and resource utilization across:

1. Scalar in-order (1 instruction/cycle)

2. Superscalar in-order (2-wide issue)

3. Superscalar out-of-order with speculative execution

4. SMT (2 threads sharing a superscalar core)

"""

class Instr:

def __init__(self, tid, text, dst=None, src=None, ready_cycle=0):

self.tid = tid                # thread id

self.text = text

self.dst = dst

self.src = src or []

self.ready_cycle = ready_cycle  # cycle when operands become available

def scalar_inorder(program):

return len(program)  # 1 instruction retired per cycle, no overlap

def superscalar_inorder(program, width=2):

"""In-order issue, but must stall on a RAW dependency within the issue group."""

cycles = 0

i = 0

while i < len(program):

cycles += 1

issued = 0

j = i

produced_this_cycle = set()

while j < len(program) and issued < width:

ins = program[j]

if any(s in produced_this_cycle for s in ins.src):

break  # dependency within the same cycle -> stop issuing more

produced_this_cycle.add(ins.dst)

issued += 1

j += 1

i = j

return cycles

def superscalar_ooo_speculative(program, width=2, window=4):

"""Out-of-order issue: any 'width' independent, ready instructions in the

instruction window can issue together, regardless of program order,

as long as their source operands are already available (renamed registers

remove false dependencies). Branches are executed speculatively using

the predicted path so dependent instructions don't have to wait."""

remaining = list(program)

all_dsts = {ins.dst for ins in program}

# architectural registers that are inputs (never produced within this

# window) are already available at cycle 0

completed = {s for ins in program for s in ins.src if s not in all_dsts}

cycles = 0

while remaining:

cycles += 1

ready = [ins for ins in remaining

if all((s in completed or s is None) for s in ins.src)]

issue_now = ready[:width]

for ins in issue_now:

completed.add(ins.dst)

remaining.remove(ins)

if not issue_now:  # avoid infinite loop safety net

issue_now = remaining[:1]

completed.add(remaining[0].dst)

remaining.pop(0)

return cycles

def smt_shared_core(threads, width=2):

"""SMT: each cycle the core can issue 'width' instructions, drawn from

ANY ready thread, so stalls/dependency bubbles in one thread are filled

by independent instructions from the other thread(s), keeping the

execution units busy."""

queues = [list(t) for t in threads]

completed = []

for t in threads:

dsts = {ins.dst for ins in t}

completed.append({s for ins in t for s in ins.src if s not in dsts})

cycles = 0

total = sum(len(t) for t in threads)

issued_count = 0

while issued_count < total:

cycles += 1

slots = width

# round-robin across threads to fill issue slots

t_idx = 0

progressed = False

while slots > 0 and any(queues):

q = queues[t_idx % len(queues)]

if q:

ins = q[0]

if all((s in completed[t_idx % len(queues)] or s is None) for s in ins.src):

completed[t_idx % len(queues)].add(ins.dst)

q.pop(0)

slots -= 1

issued_count += 1

progressed = True

t_idx += 1

if t_idx % len(queues) == 0 and not any(len(q) for q in queues):

break

if t_idx > 100 + len(threads):  # safety

break

if not progressed:

# nobody could issue (all blocked) -> issue oldest instr from any thread to progress

for qi, q in enumerate(queues):

if q:

ins = q.pop(0)

completed[qi].add(ins.dst)

issued_count += 1

break

return cycles

if __name__ == "__main__":

# A mix of independent and dependent instructions (single thread workload)

program = [

Instr(0, "I1: R1=R2+R3",  dst="R1", src=["R2", "R3"]),    # independent

Instr(0, "I2: R4=R1+R5",  dst="R4", src=["R1", "R5"]),    # depends on I1

Instr(0, "I3: R6=R7+R8",  dst="R6", src=["R7", "R8"]),    # independent

Instr(0, "I4: R9=R6+R10", dst="R9", src=["R6", "R10"]),   # depends on I3

Instr(0, "I5: R11=R2+R3", dst="R11", src=["R2", "R3"]),   # independent

Instr(0, "I6: R12=R7+R8", dst="R12", src=["R7", "R8"]),   # independent

]

scalar_cycles = scalar_inorder(program)

ss_inorder_cycles = superscalar_inorder(program, width=2)

ss_ooo_cycles = superscalar_ooo_speculative(program, width=2)

print("== THROUGHPUT COMPARISON (single thread, 6 instructions) ==")

print(f"Scalar in-order (1-wide)                : {scalar_cycles} cycles, "

f"throughput = {6/scalar_cycles:.2f} IPC")

print(f"Superscalar in-order (2-wide)            : {ss_inorder_cycles} cycles, "

f"throughput = {6/ss_inorder_cycles:.2f} IPC")

print(f"Superscalar OoO + speculative (2-wide)   : {ss_ooo_cycles} cycles, "

f"throughput = {6/ss_ooo_cycles:.2f} IPC")

# Two independent threads for SMT, each with dependency chains that would

# otherwise stall a single-thread superscalar core

thread_a = [

Instr(0, "A1: R1=R2+R3", dst="R1", src=["R2", "R3"]),

Instr(0, "A2: R4=R1+R1", dst="R4", src=["R1"]),   # depends on A1

Instr(0, "A3: R5=R4+R1", dst="R5", src=["R4", "R1"]),  # depends on A2

]

thread_b = [

Instr(1, "B1: R1=R2+R3", dst="R1", src=["R2", "R3"]),

Instr(1, "B2: R4=R1+R1", dst="R4", src=["R1"]),

Instr(1, "B3: R5=R4+R1", dst="R5", src=["R4", "R1"]),

]

ss_ooo_thread_a_alone = superscalar_ooo_speculative(thread_a, width=2)

smt_cycles = smt_shared_core([thread_a, thread_b], width=2)

print("\n== SMT vs SINGLE-THREAD (dependency-heavy chains, 3 instr each thread) ==")

print(f"Superscalar OoO, thread A alone (2-wide) : {ss_ooo_thread_a_alone} cycles "

f"(bubbles caused by RAW chain within the single thread)")

print(f"SMT, threads A+B sharing core (2-wide)   : {smt_cycles} cycles for "

f"{len(thread_a)+len(thread_b)} total instructions "

f"-> throughput = {(len(thread_a)+len(thread_b))/smt_cycles:.2f} IPC")

print("SMT fills the issue slots left idle by thread A's dependency stalls "

"with independent, ready instructions from thread B, improving "

"resource (functional unit) utilization.")

## Sample Output

== THROUGHPUT COMPARISON (single thread, 6 instructions) ==

Scalar in-order (1-wide)                : 6 cycles, throughput = 1.00 IPC

Superscalar in-order (2-wide)            : 4 cycles, throughput = 1.50 IPC

Superscalar OoO + speculative (2-wide)   : 3 cycles, throughput = 2.00 IPC

== SMT vs SINGLE-THREAD (dependency-heavy chains, 3 instr each thread) ==

Superscalar OoO, thread A alone (2-wide) : 3 cycles (bubbles caused by RAW chain within the single thread)

SMT, threads A+B sharing core (2-wide)   : 3 cycles for 6 total instructions -> throughput = 2.00 IPC

SMT fills the issue slots left idle by thread A's dependency stalls with independent, ready instructions from thread B, improving resource (functional unit) utilization.

## Result & Analysis

- For the 6-instruction stream, scalar in-order took 6 cycles (1.00 IPC); 2-wide in-order superscalar improved this to 4 cycles (1.50 IPC) but still stalled once because a dependent instruction could not issue in the same group as its producer.

- 2-wide out-of-order + speculative issue completed the same stream in only 3 cycles (2.00 IPC) by reordering independent instructions ahead of the still-pending dependent ones, fully using both issue slots every cycle.

- Running the dependency-chain thread (A) alone on the OoO core still took 3 cycles for only 3 instructions (1.00 IPC) because every instruction in that chain depends on the previous one, leaving one issue slot idle each cycle.

- Adding a second, equally-dependent thread (B) via SMT let the core finish both threads' 6 instructions together in the same 3 cycles (2.00 IPC) - the idle issue slot in thread A's dependency chain was filled by ready instructions from thread B, doubling resource utilization without needing any extra functional units.

- Overall ranking observed: Scalar < In-order superscalar < Out-of-order superscalar ≈ SMT (on a dependency-bound single thread, SMT recovers the throughput lost to bubbles).

# Overall Conclusion

Across the five experiments, the progression from basic register-transfer/ALU operations to a full 5-stage pipeline, then to hazard-aware pipelining, and finally to superscalar out-of-order and SMT execution demonstrates how modern processors extract instruction-level and thread-level parallelism:

- Register transfer and ALU operations (Exp. 1) are the fundamental building blocks of every datapath operation.

- Multi-bus organization (Exp. 2) removes the single-resource bottleneck of a shared bus, giving up to 3x more transfer throughput in our model.

- Pipelining (Exp. 3) overlaps instruction execution to raise throughput close to the number of pipeline stages (up to 5x speedup) without changing per-instruction latency.

- Hazards (Exp. 4) are the practical limiters of pipelining; forwarding, stalling and branch prediction recover most, but not all, of the ideal pipeline speedup.

- Superscalar, out-of-order, speculative execution and SMT (Exp. 5) push performance further by issuing multiple instructions per cycle, reordering around stalls, and filling idle issue slots with work from other threads, improving both instruction throughput and functional-unit utilization.